# PCB Defect Detection – Defects vs Suitable Algorithms Reference Sheet

This document lists common Printed Circuit Board (PCB) manufacturing defects and maps them to suitable computer vision and deep learning algorithms for Automated Optical Inspection (AOI).

| Defect Type | Description | Possible Algorithms |
| --- | --- | --- |
| Open Circuit | Break in copper trace causing discontinuity. | Image subtraction, Canny edge detection, CNN, YOLO |
| Short Circuit | Unwanted connection between tracks. | Connected component labeling, CNN, YOLO |
| Missing Hole | Hole not drilled or incomplete. | Hough circle transform, blob detection |
| Mouse Bite | Small copper missing on edges. | Edge detection, contour analysis, DL classification |
| Spurious Copper | Extra copper present. | Segmentation networks, U-Net |
| Scratch | Elongated mechanical damage. | Texture filters, U-Net segmentation |
| Crack | Thin fracture in trace. | High resolution edge detection, hybrid CV+DL |

# Literature Insights Used for Algorithm Selection

## Hybrid Vision + Deep Learning (Cracks & Scratches) (https://www.sciencedirect.com/science/article/pii/S2590123025001550#fg0020)

Research shows that very thin defects such as cracks and scratches are difficult to detect using only deep learning because lighting variations and surface texture changes confuse the model.

Defects Algo

Working Principle (Simplified):

- First align PCB images using feature matching (ORB + RANSAC)

- Subtract aligned images to isolate suspicious regions

- Remove noise using morphological filtering

- Apply U-Net only on suspected areas for pixel-level defect segmentation

Key Insight for AOI System

Use traditional computer vision for localization + deep learning for confirmation.

Best suited for:

- Cracks

- Fine scratches

- Hairline copper breaks

Why this matters

- Works with small datasets

- Robust to lighting changes

- Reduces false positives

- Faster than running deep learning everywhere

## Real-Time Industrial Detection (Multiple Small Defects) (https://www.nature.com/articles/s41598-025-20387-x#Sec3)

Modern PCB inspection favours single-stage object detectors because inspection must run on production lines without delay.

Improved YOLO-based models enhance:

- small defect detection

- edge clarity

- speed

Best suited for:

- Missing hole

- Mouse bite

- Burr

- Short circuit

- False copper

- Open circuit

Key Insight

Use YOLO-type object detection when defects are visible patterns rather than microscopic fractures.

Why this matters

- Real-time detection

- Detects multiple defects in one pass

- Good for mass production inspection

## 3. Multi-Class Defect Classification (Full AOI Systems) (https://www.mdpi.com/2076-3417/15/24/13115)

CNN-based detection systems can classify many PCB defects simultaneously using one trained model.

Typical detectable defects:

- Open circuit

- Short circuit

- Missing hole

- Spur / spurious copper

- Dent damage

- Etching defects

- Pin holes

- Scratches

FIGURES: