# PCB Defect Detection – Defects vs Suitable Algorithms Reference Sheet

This document lists common Printed Circuit Board (PCB) manufacturing defects and maps them to suitable computer vision and deep learning algorithms for Automated Optical Inspection (AOI).

| Defect Type | Description | Industrial Influence | Detection Challenge | Examples |
| --- | --- | --- | --- | --- |
| Short | Unintended conductive bridge. | Leads to unintended current flow. | Subtle conductive paths may blend with normal traces. |  |
| Open | Broken trace. | Causes electrical discontinuity. | Requires high-resolution imaging to detect thin breaks in traces. |  |
| Missing Hole | Incomplete hole plating. | Disrupts circuit connectivity. | Hard to detect small or obscured holes. |  |
| Mouse Bite | Irregular board edge. | Weakens PCB structural integrity. | Small irregularities at edges are easily missed. |  |
| Spur | Metal protrusion. | Causes short circuits or damage components. | Tiny protrusions can be mistaken for noise or surface texture. |  |
| Spurious Copper | Residual copper debris. | Leads to poor connectivity. | Uneven ink coverage requires fine-grained analysis. |  |
| Cold Solder or Void Solder | Weak metallurgical bond. | Weakens joints. | Poorly formed joints require detailed inspection of 3D shapes and reflective surfaces. |  |
| Misplaced Component | Incorrect part orientation or misalignment. | Causes improper functionality. | Precise alignment checks are needed. |  |
| Pinhole | Micro-hole in solder mask. | Reduces joint strength and conductivity. | Small voids in solder joints are difficult to distinguish. |  |
| Bridging | Solder/copper overlap. | Causes malfunctions and potential overheating risks. | Demands high spatial resolution and accurate edge detection. |  |
| Excessive Solder | Overflow solder. | Causes uneven surfaces and leads to short circuits. | Convex shapes need precise height/shape analysis. |  |
| Wiring Defect | Incorrect trace width/spacing. | Causes functional failures. | Requires multi-scale feature extraction. |  |
| Missing Component | Unpopulated part. | Results in incomplete functionality. | Difficulty in distinguishing whether is missing component or misassembled. |  |
| Cosmetic Defect | Surface scratch/discoloration. | Affects product aesthetics. | Surface scratches or dirt are often visually similar to normal patterns. |  |

# Literature Insights Used for Algorithm Selection

## Hybrid Vision + Deep Learning (Cracks & Scratches) (https://www.sciencedirect.com/science/article/pii/S2590123025001550#fg0020)

Research shows that very thin defects such as cracks and scratches are difficult to detect using only deep learning because lighting variations and surface texture changes confuse the model.

Defects Algo

Working Principle (Simplified):

- First align PCB images using feature matching (ORB + RANSAC)

- Subtract aligned images to isolate suspicious regions

- Remove noise using morphological filtering

- Apply U-Net only on suspected areas for pixel-level defect segmentation

Key Insight for AOI System

Use traditional computer vision for localization + deep learning for confirmation.

Best suited for:

- Cracks

- Fine scratches

- Hairline copper breaks

Why this matters

- Works with small datasets

- Robust to lighting changes

- Reduces false positives

- Faster than running deep learning everywhere

## Real-Time Industrial Detection (Multiple Small Defects) (https://www.nature.com/articles/s41598-025-20387-x#Sec3)

Modern PCB inspection favours single-stage object detectors because inspection must run on production lines without delay.

Improved YOLO-based models enhance:

- small defect detection

- edge clarity

- speed

Best suited for:

- Missing hole

- Mouse bite

- Burr

- Short circuit

- False copper

- Open circuit

Key Insight

Use YOLO-type object detection when defects are visible patterns rather than microscopic fractures.

Why this matters

- Real-time detection

- Detects multiple defects in one pass

- Good for mass production inspection

## 3. Multi-Class Defect Classification (Full AOI Systems) (https://www.mdpi.com/2076-3417/15/24/13115)

CNN-based detection systems can classify many PCB defects simultaneously using one trained model.

Typical detectable defects:

- Open circuit

- Short circuit

- Missing hole

- Spur / spurious copper

- Dent damage

- Etching defects

- Pin holes

- Scratches